These are all the Inputs and Outputs on the system <PLC Tag> [I/O location]:

- Discrete Inputs

  - E-Stop / Master Control Relay <MCR_FB> [In 0]

  - Loss of enclosure Alarm relay <CR17_LOSS_OF_PRESSURE_PURGE> [In 1]

  - Reset Pushbutton panel door <ALARM_RESET> [In 3]

  - PSA Speed Rotation Switch Proximity Switch Stage 1? <_05_ZSL_002> [In 4]

  - PSA Speed Rotation Switch Proximity Switch Stage 2? <_05_ZSL_004> [In 5]

  - Feed Gas to PSA Position Switch Open <_05_ZSO_010> [In 8]

  - Feed Gas to PSA Position Switch Closed <_05_ZSC_010> [In 9]

  - Drain Valve Position Switch Open (incorrectly labeled in logic as ‘Feed Gas to PSA Slow Pressurization Valve Position Switch Open’) <_05_ZSO_015> [In 10]

  - Drain Valve Position Switch Closed (incorrectly labeled in logic as ‘Feed Gas to PSA Slow Pressurization Valve Position Switch Closed’) <_05_ZSC_015> [In 11]

  - Product Gas to Client Position Switch Open <_05_ZSO_032> [In 12]

  - Product Gas to Client Position Switch Closed <_05_ZSC_032> [In 13]

- Discrete Outputs

  - CR1 Feed Valve is open <CR_05_XV_010> [Out 0] wired to correctly to send our PLC a signal

  - CR2 Drain Valve is open (incorrectly labeled ‘PSA System is tripped’) <CR_05_XV_015> [Out 1] wired to the terminals that are supposed to send our PLC a signal saying the PSA has tripped

  - CR3 PSA Product Valve is open <PSA_TRIP> [Out 2] never used in the programming and wired to the terminals that are supposed to send our PLC a signal saying the product valve is open

  - CR4 PSA Product Valve is open <CR_05_XV_032> [Out 3] wired to CR4, we do not get a signal from this

  - Amber light indicator, common alarm <AMBER_LIGHT> [Out 10]

  - Red light indicator, common shutdown <RED_LIGHT> [Out 11]

- Analog Inputs

  - 05PT005 Pressure Transmitter (Stage 1 bottom pressure) <_05_PIT_005_RAW> [A In 0]

  - 05PT008 Pressure Transmitter (Stage 2 inlet pressure) <_05_PIT_008_RAW> [A In 1]

  - 05PT009 Pressure Transmitter (Stage 2 bottom pressure) <_05_PIT_009_RAW> [A In 2]

Analog Input Scaling and Alarms

- 05-PIT-005 Stage 1 bottom pressure <_05_PIT_005>

  - Range: 0 to 300 psig

  - Alarms: HH / H / L

  - Alarm Setpoints: HH 270 / H 255 / L 225

- 05-PIT-008 Stage 2 inlet pressure <_05_PIT_008>

  - Range: 0 to 300 psig

  - Alarms: HH / H / L / LL

  - Alarm Setpoints: HH 0 / H 0 / L 0 / LL 0

- 05-PIT-009 Stage 2 bottom pressure <_05_PIT_009>

  - Range: 0 to 300 psig

  - Alarms: HH / H / L / LL

  - Alarm Setpoints: HH 0 / H 0 / L 0 / LL 0

Alarms <Trip Tag> {Alarm Bit}

- Interlocks <PSA_ALARM[0]> and <PSA_ALARM[1]>

  - E-Stop <MCR_FB> {PSA_ALARM[0].0}

  - PAHH-005 High High Stage 1 bottom pressure (if it exists – it does) <_05_PIT_005.HiHi_Alm> {PSA_ALARM[0].1}

  - PALL-005 Low Low Stage 1 bottom pressure (if it exists – it does NOT) {PSA_ALARM[0].4}

  - PAHH-008 High High Stage 2 inlet pressure (if it exists – it does) <_05_PIT_008.HiHi_Alm> {PSA_ALARM[0].5}

  - PALL-008 Low Low Stage 2 inlet pressure (if it exists – it does) <_05_PIT_008.LoLo_Alm> {PSA_ALARM[0].8}

  - PAHH-009 High High Stage 2 bottom pressure (if it exists – it does) <_05_PIT_009.HiHi_Alm> {PSA_ALARM[0].9}

  - PALL-009 Low Low Stage 2 bottom pressure (if it exists – it does) <_05_PIT_009.LoLo_Alm> {PSA_ALARM[0].12}

  - ZAO-010 Feed Valve fail to open <_05_XV_010_FailToOpenAlm> {PSA_ALARM[1].0}

  - ZAC-010 Feed Valve fail to close <_05_ XV_010_FailToCloseAlm> {PSA_ALARM[1].1}

  - ZAO-015 Drain Valve fail to open <_05_ XV_015_FailToOpenAlm> {PSA_ALARM[1].2}

  - ZAC-015 Drain Valve fail to close <_05_ XV_015_FailToCloseAlm> {PSA_ALARM[1].3}

  - ZAO-032 Product Valve fail to open <_05_ XV_032_FailToOpenAlm> {PSA_ALARM[1].4}

  - ZAC-032 Product Valve fail to close <_05_ XV_032_FailToCloseAlm> {PSA_ALARM[1].5}

  - MA-01 VFD 1 Fault <VFD1.Faulted_Alm> {PSA_ALARM[1].6}

  - MA-02 VFD 2 Fault <VFD2.Faulted_Alm> {PSA_ALARM[1].7}

  - YA-017 Loss of pressure purge <CR17_LOSS_OF_PRESSURE_PURGE aka Local:1:I:Data.1> {PSA_ALARM[1].8}

  - Disabled, but there was an alarm if the Stage 1 Prox switch didn’t change states for 10 seconds when Stage 1 VFD was told to run in Auto {PSA_ALARM[1].9}

  - Disabled, but there was an alarm if the Stage 2 Prox switch didn’t change states for 10 seconds when Stage 2 VFD was told to run in Auto {PSA_ALARM[1].10}

- Common Alarms <PSA_ALARM[2]> (PSA_ALARM[3] exists but is not used)

  - PAH-005 High Stage 1 bottom pressure (if it exists – it does) <_05_PIT_005.Hi_Alm> {PSA_ALARM[2].0}

  - PAL-005 Low Stage 1 bottom pressure (if it exists – it does) <_05_PIT_005.Lo_Alm> {PSA_ALARM[2].1}

  - PAH-008 High Stage 2 inlet pressure (if it exists – it does) <_05_PIT_008.Hi_Alm> {PSA_ALARM[2].2}

  - PAH-008 Low Stage 2 inlet pressure (if it exists – it does) <_05_PIT_008.Lo_Alm> {PSA_ALARM[2].3}

  - PAH-009 High Stage 2 bottom pressure (if it exists – it does) <_05_PIT_009.Hi_Alm> {PSA_ALARM[2].4}

  - PAH-009 Low Stage 2 bottom pressure (if it exists – it does) <_05_PIT_009.Lo_Alm> {PSA_ALARM[2].5}

There are five different sequences of operation:

- G100 Normal Start

- G200 Short Stop

- G300 Long Stop

- G400 System Trip

- G500 Service

G100 Normal Start Sequence

- Standard sequence when we want to operate the PSA

- Step 100

  - Initial step, will not advance unless operation action is taken

  - Any of these conditions will reset G100 Sequence to this initial step:

    - PSA E-Stop pressed

    - Any step in the Trip Sequence active besides the initial step (400)

    - Normal Start hasn’t finished and of the following:

      - Short Stop Sequence button pressed on HMI

      - Long Stop Sequence button pressed on HMI

      - System Trip Sequence triggered by a button pressed on HMI or any interlock alarm coming in

  - SeqNormalStart.Available has to be met to start the Normal Start Sequence, these are the conditions:

    - All 5 sequences are on their first step (100,…,500)

    - Both VFDs stopped and ready

    - No interlock alarms PSA_ALARM[0] and PSA_ALARM[1]

  - To transition to the next step, the Normal Start Sequence button has to be pressed on the HMI

- Step 110

  - This puts the VFDs in Local control (controlled by the PLC by setting VFD1.Mode and VFD2.Mode to 2), and gives them a command to start moving

    - I do not know where the VFD speed setting comes from, guessing the HMI

  - To transition to the next step, the HMI start button has to be off, which was taken care of with the previous transition

- Step 120

  - This puts all 3 valves into Auto control, but does not tell them to open

  - This puts all 3 valves and both VFDs into AutoModeLock

    - No idea what this does, I assume it’s used in the HMI because it does nothing in the PLC – will treat it like it locks everything in Auto/Local until released

  - To transition to the next step, all 3 valves have to be in Auto and Closed, and both VFDs have to be in Local and Stopped (seems in conflict with previous step as there is no ONDelay value BUT there might be a speed setting of zero)

- Step 130

  - This opens the Drain Valve and opens the Product Valve

  - Keeps the VFDs in Local and tells them to run with a speed setting

  - To transition to the next step, the Drain Valve and Product Valve have to be open and both VFDs have to be running at speed

- Step 135

  - This keeps the Drain Valve open and Product Valve open

  - Keeps the VFDs in Local and tells them to run with a speed setting

  - To transition to the next step, Stage 2 Inlet Pressure has to be above 150 psig continuously for 50 minutes (will need to pressurize the system slowly with the needle valve otherwise we’re stuck)

- Step 140

  - This opens the Feed Valve and keeps the Drain Valve open and the Product Valve open

  - Keeps the VFDs in Local and tells them to run with a speed setting

  - To transition to the next step, the Feed Valve has to be open

- Step 150

  - This keeps the Feed Valve open, the Drain Valve open, and the Product Valve open

  - Keeps the VFDs in Local and tells them to run with a speed setting

  - To transition to the next step, Stage 2 VFD has to be running and the Product Valve has to be open

- Step 160

  - AutoModeLock is released for the valves and VFDs

  - If in Auto, the Feed Valve, Drain Valve and Product Valve will all be open

  - Both VFDs are put into Remote Control mode (VFD1.Mode and VFD2.Mode = 0)

  - VFDs will run with a speed setting that the operator is in control of

  - To transition to the next step, both VFDs have to be in Remote Control mode

- Step 170

  - This should be the cruising speed step during operation

  - If in Auto, the Feed Valve, Drain Valve and Product Valve will be open

  - VFDs will be running with an operator-controlled speed setting

  - To transition back to the first step (100), any of the following will have to happen:

    - Short Stop Sequence button pressed on HMI

    - Long Stop Sequence button pressed on HMI

    - System Trip Sequence initiated (button or any interlock alarm)

G200 Short Stop Sequence

- Used more for a short outage, maybe a few hours to a day or two?

- Step 200

  - Initial step, will not advance unless Short Stop Sequence button is pressed on the HMI

  - Either of the following conditions will one-shot reset this sequence to this first step:

    - Short Stop Sequence has not completed and the System Trip Sequence Trigger is not active

    - Any step in the Trip Sequence active besides the initial step (400)

  - SeqShortStop.Available has to be met to start the Short Stop Sequence, these are the conditions:

    - All other sequences are on their first step (100, 300, 400, 500)

  - To transition to the next step, the Short Stop Sequence button has to be pressed on the HMI

- Step 210

  - Sends an Auto Open command to all three valves (does not put them in Auto though)

  - Sends an Auto Start command to both VFDs

  - To transition to the next step, the Normal Start Sequence has to be on Step 100 and the Short Stop Sequence button on the HMI has to be off (unlatched by previous transition)

- Step 220

  - Puts all 3 valves into Auto Mode and opens them

  - Puts both VFDs in Local control and tells them to run

  - To transition to the next step, all 3 valves need to be in Auto Mode and both VFDs need to be in Local control

- Step 230

  - Keeps all 3 Valves open

  - Keeps both VFDs running via Auto Start Command, but no speed setting?

  - Not really anything needed to transition to the next step

- Step 240

  - Keeps only the Drain Valve open with Auto Open Command

  - Keeps both VFDs running via Auto Start Command, but no speed setting?

  - To transition to the next step, all 3 valves need to not be open (seems in direct conflict with the Drain Valve Auto Open Command)

- Step 250

  - Auto Open Command for the Drain Valve

  - Keeps both VFDs running via Auto Start Command, but no speed setting?

  - To transition to next step, both VFDs need to be stopped

- Step 260

  - Last step in sequence

  - Auto Open Command for the Drain Valve

  - Keeps both VFDs running via Auto Start Command, but no speed setting?

  - To transition back to the first step, the Reset Sequence button has to be pushed on the HMI

G300 Long Stop Sequence

- Used for longer term outages

- Step 300

  - Initial step, will not advance unless Long Stop Sequence button is pressed on HMI

  - Either of the following conditions will reset this sequence to this first step:

    - Long Stop Sequence has not completed and the System Trip Sequence Trigger is not active

    - Any step in the Trip Sequence active besides the initial step (400)

  - SeqLongStop.Available has to be met to start the Long Stop Sequence, these are the conditions:

    - All other sequences are on their first step (100, 200, 400, 500)

  - To transition to the next step, the Long Stop Sequence button has to be pressed on the HMI

- Step 310

  - Gives all 3 valves an Auto Open Command

  - Gives both VFDs an Auto Start Command and a speed setting

  - To transition to the next step, the Normal Start Sequence has to be on Step 100 and the Long Stop Sequence button on the HMI has to be off (unlatched by previous transition)

- Step 320

  - Puts all 3 valves in Auto and opens them

  - Puts both VFDs in Local control and tells them to run with a speed setting

  - To transition to the next step, all 3 valves need to be in Auto and both VFDs need to be in Local control

- Step 330

  - Gives all 3 valves an Auto Open Command

  - Tells both VFDs to run with a speed setting

  - To transition to the next step, both VFDs have to be running at speed

- Step 340

  - Only keeps the Drain Valve open

  - Tells both VFDs to run with a speed setting

  - To transition to the next step, the Feed Valve and Product Valve cannot be open

- Step 350

  - Only keeps the Drain Valve open

  - Tells both VFDs to run with a speed setting

  - To transition to the next step, the Stage 2 inlet pressure needs to be less than or equal to 10 psig continuously for 5 minutes

- Step 360

  - Only keeps the Drain Valve open

  - Both VFDs stopped

  - To transition to the next step, there are a few possibilities

    - Both VFDs need to be stopped continuously for 15 minutes, will move on to Step 370 (don’t think this will ever happen)

    - Both VFDs stopped and Stage 2 inlet pressure less than or equal to 10 psig will move on to Step 370

    - Both VFDs stopped and Stage 2 inlet pressure greater than 10 psig will go back to Step 371 (which loops back to Step 350)

- Step 370

  - Last step in sequence

  - Only keeps the Drain Valve open

  - Reset Sequence button has to be pushed on the HMI to go back to Step 300

- Step 371

  - Only keeps the Drain Valve open

  - Tells both VFDs to run with a speed setting

  - To transition back to Step 350, both VFDs need to be at speed

G400 System Trip Sequence

- This is what happens when something goes real wrong

- Step 400

  - Initial step

  - The only thing that resets this sequence to the first step is the Reset Tripped Sequence button on the HMI

  - Transition requires any of the following:

    - System Trip Sequence button pressed on the HMI

    - Interlock alarm PSA_ALARM[0] or PSA_ALARM[1]

- Step 410

  - All 3 valves put into Auto but are closed

  - Both VFDs put into Local control, but are stopped

  - To transition to the next step, the System Trip Sequence button on the HMI has to be off (unlatched by previous transition step), the System Trip Sequence trigger has to be off (unlatched by previous transition step), all 3 valves in Auto Mode and both VFDs in Local control

- Step 420

  - All 3 valves put in AutoModeLock – still closed

  - Both VFDs put in AutoModeLock – still stopped

  - To transition back to the first step, both VFDs need to be stopped, all 3 valves need to be closed and the Reset Tripped Sequence button on the HMI needs to be pressed

G500 Service Sequence

- Used when you want to service the PSA – would recommend killing main power probably

- Step 500

  - Initial step

  - PSA E-Stop being pressed resets the Service Sequence to Step 500, or any step in the Trip Sequence (G400) active besides the initial step (400)

  - To transition to the next step, the Service Mode button has to be pressed on the HMI

- Step 510

  - This will just sit here unless operator action is taken

  - AutoModeLocks are turned off for all 3 valves and both VFDs

  - To transition to the next step, the Reset Service Mode button has to be pressed on the HMI, the Service Mode button has to be off (was unlatched in previous transition), the AutoModeLocks are off for all 3 valves and both VFDs

- Step 520

  - AutoModeLocks are turned on for all 3 valves and both VFDs

  - All 3 valves are put into Auto

  - Both VFDs are put into Local control

  - To transition back to the first step, AutoModeLock has to be on for all 3 valves and both VFDs, all 3 valves are in Auto, both VFDs are in Local control, and the Reset Service Mode button is off (unlatched in previous transition step)

Valve controls

There is a built-in AOIs (add-on instructions) that Xebec made that controls how the valves operate. Each valve is controlled by its own AOI (Xebec_ValveOnOff), so they all operate in an identical manner.

Position:

OpenFB (open feedback) is tied to the Open Position Switch input for the valve, and CloseFB (closed feedback) is tied to the Closed Position Switch input for the valve.

Interlock:

Functionally bypassed, as the logic will always say the interlock is okay. There’s also a bit to configure whether an interlock exists or not.

Opening/Closing:

There’s an ONDelay (delays the valve opening for a specified time) and an OFFDelay (delays the valve closing for a specified time), but they are all set to 0 so they don’t have any effect. There is also a FaultDelay that’s set to 5.000 seconds. This FaultDelay is used to as a timer before declaring that the valve is in the wrong position based on what it is told to do via the Output tag (tied to the discrete output for the valve). The valve can have a Fail To Open Alarm (FailToOpenAlm) or a Fail To Close Alarm (FailToCloseAlm). These are system interlocks for the PSA. So if the valve is told to open and doesn’t hit the open position switch in 5 seconds, the Fail To Open Alarm comes in.

The valve can be operated in Manual (CMDMan) or Auto (CMDAuto). Mode having a value of 1 means the valve is in Auto.

State:

There are 6 possible states.

0 – Closed

1 – Open

2 – Fail To Open

3 – Fail To Close

4 – Sensor Failure (both position switches on at same time)

5 – Interlock Fault (valve is Interlocked, not possible with how it’s currently programmed)

VFD Controls

There is also an AOI for the VFD control (Xebec_VFDCtrl). Both VFDs are controlled by their own instance of the AOI.

Position:

There is a prox switch to detect motion with the gear teeth. It’s not part of the AOI, and it is not really used in the programming. Speed is not calculated by it, and no alarms will trigger.

Interlock:

This is configured and only tied to the E-Stop button on the PSA panel. If the E-Stop is pressed, the VFDs will be interlocked in the logic.

Running:

Each VFD can be run in Local control (Mode = 2, PLC is in charge of making it move) or Remote control (Mode = 0, operator is in charge). These also have ONDelay and OFFDelay, but the value is 0. There is also a FaultDelay of 5.000 seconds which starts when the Faulted Input signal from the VFD comes on, or the Ready signal from the VFD is not on. The Faulted Input signal from the VFD prevents it from running.

It’s unclear to me where the Local control speed setting values come for how fast it should be spinning. I imagine we will want to control the Remote control speed settings based on how much gas flow or a specific component flow is going into the PSA.

State:

There are 6 possible states.

0 – Equipment Stopped (No Start Command given and Output to VFD = 0)

1 – Equipment Running (Start command given and Output to VFD > 1)

2 – Fail To Stop (No Start Command is given, but Output to VFD > 5 for 15.000 seconds straight)

3 – Fail To Start (Start Command given, but Output to VFD < 1 for 5.000 seconds straight)

4 – VFD Faulted (Faulted Input signal from VFD or a lack of VFD Ready signal for 5 seconds)

5 – Interlock Fault (since Interlock is configured, comes on when the Interlock is not ok which is only triggered by pressing the PSA E-Stop)

Miscellaneous notes:

I think the programming to light up the red light on the panel is incorrect.

I think the Short Stop Sequence might get hung up because of the Drain Valve.

Alarms setpoints are going to need to reconfigured for PIT-005, and actual non-zero values will need to be put in for PIT-008 and PIT-009. Otherwise there will be constant interlocks.

Might be wise to calculate rotation speed based on gear teeth, and possibly alarm if it doesn’t move in a certain amount of time.

2 of the 3 signals that are supposed to get sent to us are incorrect.

I am not sure how the drain works on this, but that valve is open more than the others during the sequence. Will we be venting syngas during this?