PSA

Overview

The Pressure Swing Adsorption (PSA) unit separates purified hydrogen (H2) gas from the syngas fed to it. One product stream produced is called the Tail Gas and it is comprised of hydrogen-depleted syngas. The other product stream is the Purified Product, which is high purity H2.

| Tag | Description | Logged | Alarmed | Display |
| --- | --- | --- | --- | --- |
| XX-#### | PSA Enable |  |  | Yes |
| XX-#### | PSA E-Stop |  |  | Yes |
| XX-#### | PSA VFD Enable |  |  | Yes |
| XX-#### | PSA Feed Valve |  |  | Yes |
| XX-#### | PSA Product Valve |  |  | Yes |
| XX-#### | PSA Motor Start |  |  | Yes |
| XX-#### | PSA VFD Running |  |  | Yes |
| XX-#### | PSA Motor Overtemp |  |  | Yes |
| XX-#### | PSA Gear Detector |  |  | Yes |
| XX-#### | PSA VFD Speed Control |  |  | Yes |
| XX-#### | PSA Feed Flow | Yes |  | Yes |
| XX-#### | PSA Product Flow | Yes |  | Yes |

Calculations

PSA Rotational Speed

RPH = (Time2 – Time1) * TeethNo

Where:

Time2 = timer value the instant the second tooth is detected

Time1 = timer value the instant the first tooth is detected

TeethNo = number of teeth on gear (32)

*Note: preferred rotational speed in rotations per hour (RPH) so conversion will be necessary (conversion depends on PLC capabilities)

Averaged PSA Feed Flow

AveFeedFlow = (T1Flow + T2Flow + … T60Flow)/60

Where:

T1Flow = Feed Flow at most recent second

T2Flow = Feed Flow at 2nd most recent second

…

T60Flow = Feed Flow at sixtieth most recent second

[Gives a 1-minute rolling average of the PSA Feed Flow since Feed Flow will fluctuate due to nature of PSA operation (lb/hr)]

Averaged PSA Feed SCFM (0C, 1atm)

AveFeedSCFM = AveFeedFlow * SyngasMW * R * P,s * T,s

Where:

AveFeedFlow (lb/hr) = 1-minute rolling average of PSA Feed Flow

Sygngas MW (lb/lbmol) = MW of nearest known syngas – calculated by composition

R = 1.31443 ft3*atm/lbmol/K

P,s = 1 atm

T,s = 273.15 K

Minimum PSA Rotational Speed by Flow

MinRPH,Flow = AveFeedSCFM * NominalRPH / NominalFlow

Where:

AveFeedSCFM = 1-minute rolling average of PSA Feed Flow in SCFM

NominalRPH = nominal PSA Rotational Speed at nominal feed flow

NominalFlow = nominal PSA Feed Flow

Minimum PSA Rotational Speed by CO

MinRPH,CO = AveFeedSCFM * CO%/100 * NominalRPH / NominalCO

Where:

AveFeedSCFM = 1-minute rolling average of PSA Feed Flow in SCFM

CO% = CO composition in feed syngas

NominalRPH = nominal PSA Rotational Speed at nominal feed flow

NominalCO = nominal PSA Feed CO based on design

Minimum PSA Rotational Speed by CO2

MinRPH,CO2 = AveFeedSCFM * CO2%/100 * NominalRPH / NominalCO2

Where:

AveFeedSCFM = 1-minute rolling average of PSA Feed Flow in SCFM

CO2% = CO2 composition in feed syngas

NominalRPH = nominal PSA Rotational Speed at nominal feed flow

NominalCO2 = nominal PSA Feed CO2 based on design

Minimum PSA Rotational Speed by N2

MinRPH,N2 = AveFeedSCFM * N2%/100 * NominalRPH / NominalN2

Where:

AveFeedSCFM = 1-minute rolling average of PSA Feed Flow in SCFM

N2% = N2 composition in feed syngas (calculated by subtraction)

NominalRPH = nominal PSA Rotational Speed at nominal feed flow

NominalN2 = nominal PSA Feed N2 based on design

Averaged PSA Product Flow

AveProdFlow = (T1Flow + T2Flow + …T60Flow)/60

Where:

T1Flow = Product flow at most recent second

T2Flow = Product flow at 2nd most recent second

…

T60Flow = Product flow at sixtieth most recent second

[Gives a 1-miunte rolling average of PSA Product Flow in SCFM (0C, 1atm) since Product Flow will fluctuate due to nature of PSA operation]

PSA VFD Speed Control

SpeedControlOutput = A * RPH_HMI + B

Where:

A = constant based on empirical measurements

RPH_HMI = RPH setting entered via HMI (this should have a min. value)

B = constant based on empirical measurements

[This will be dependent on VFD settings {min/max rotational settings} as well as PSA gearing – a 0% setting is a 4 mA signal and will move the PSA at the minimum VFD speed, 100% = 20 mA and will move the PSA at the maximum VFD speed.]

Interlocks

Oxygen detected on inlet to the PSA should interlock the entire PSA.

Disabling the PSA sends an E-Stop signal to the PSA which cuts control power to the cabinet and also interlocks the PSA Motor Start signal from being sent (PSA Motor will not start when PSA is disabled).

If the PSA Running signal is not received, the Feed Valve and Product Valve are both interlocked closed.

A Gear Tooth being detected for too long will one-shot the PSA Motor signal Off, which causes the PSA Running signal to stop being received, resulting in the Feed Valve and Product Valve latching closed.

A Gear Gap being detected for too long will one-shot the PSA Motor signal Off, which causes the PSA Running signal to stop being received, resulting in the Feed Valve and Product Valve latching closed.

The PSA Running signal should be detected in under 3 seconds of giving the PSA Motor Start signal. Lack of PSA Running signal for too long will one-shot the PSA Motor signal Off, which causes the PSA Running signal to stop being received, resulting in the Feed Valve and Product Valve latching closed.

The PSA VFD Ready signal not being received when the PSA is Enabled interlocks the PSA Motor Start signal Off.

The PSA Motor Overtemp signal being received when the PSA is Enabled interlocks the PSA Motor Start signal Off, which will interlock the Feed Valve and Product Valve closed.

Control Logic

Enabling the PSA, via the HMI, will stop sending the E-Stop signal to the PSA. It will also momentarily send a PSA VFD Enable signal for 2 seconds. This should enable control power to the PSA.

A short period of time after Enabling the PSA, the PSA VFD Ready signal should be received. At this point, the PSA Motor should be allowed to be started. Clicking “Enabled” on the PSA Motor will start rotation of the PSA. The PSA VFD Running signal should be received shortly after. The PSA Gear Detector should begin changing states, showing “GAP” or “TOOTH” on the HMI.

When the PSA is rotating, the rotational speed is calculated by the elapsed time between the first detection of two consecutive gear teeth. This verifies that the PSA is still rotating and shows the rate of rotation.

The PSA VFD Speed Control signal will change how fast the PSA rotates. On the HMI, the desired RPH setting is entered and a device-specific correlation will be used to calculate the required Speed Control signal for the entered RPH. Clicking the Up/Down arrow on the PSA Faceplate will change the RPH setting by 0.5. A small graph is on the PSA Motor section of the faceplate showing the calculated RPH, which should track very closely with the entered value. It may take a little bit of time upon startup or a setpoint change for the gear tooth detection to match up.

The PSA RPH should exceed each of the minimum rotational speeds calculated by Flow, CO, CO2 and N2. If not, the PSA Motor section on the Faceplate should blink red and indicate which one(s) is out of compliance and the one that is most out of compliance should have a calculated minimum rotational speed (RPH) next to it.

As soon as the PSA VFD Running signal has been received, the Feed Valve can be opened and then the Product Valve can be opened. Product Flow should be detected shortly after if the system has fully pressurized.

Provide alarm for PSA Motor Overtemp (PSA Enabled and PSA Motor Overtemp signal received).

Provide alarm for PSA Gear Fault (“GAP” too long or “TOOTH” too long).

Provide alarm for PSA VFD Run Fault (PSA Motor Start but PSA VFD Running signal not received after 3 seconds).

Provide alarm for Low PSA Rotation Speed and indicate whether it is due to Flow, CO, CO2 or N2.

PID Loops

None.